🎯 Feasible & Winnable Project Ideas for FPGA Hackathon 2026

I've analyzed the hackathon requirements against your constraints (3-week timeline, learning curve, winning potential) and identified projects that maximize your chances.

🏆 TIER 1: HIGHEST WIN PROBABILITY (Choose from these)

1. Real-Time ECG Arrhythmia Detection System ⭐ RECOMMENDED

Why This Wins:

- Clear medical urgency (life-critical application)

- Simple 1D CNN architecture (FPGA-friendly)

- Measurable performance metrics (sensitivity, specificity, latency)

- Abundant public datasets (MIT-BIH Arrhythmia Database)

- Real-time requirement is genuine (not artificial)

Technical Approach:

ML Model:

- 1D CNN with 4-5 layers

- Input: 180-sample ECG segment (1-second at 180 Hz)

- Output: Normal / Arrhythmia classification

- Model size: ~50KB weights (tiny!)

- Quantize to INT8

Hardware Acceleration:

- Accelerate the 1D convolution layers

- Systolic array for MAC operations

- Streaming architecture (process samples as they arrive)

- Latency target: <10ms per classification

FPGA Implementation:

ECG Sensor Input → ADC Interface → Preprocessing →

1D Conv Accelerator (YOUR RTL) → Classification → Alert Output

Why Judges Love This:

- Medical impact (saves lives)

- Clear bottleneck (convolution is 80% of compute)

- Real-time constraint (ICU monitoring needs instant detection)

- Demonstrates edge AI (portable bedside monitor)

Datasets:

- MIT-BIH Arrhythmia Database (free, standard benchmark)

- 48 ECG recordings, annotated

Expected Results:

- Accuracy: ~97% (state-of-the-art is ~98%)

- Latency: 5-8ms (vs. 100ms+ on CPU)

- Power: ~2W (FPGA) vs. ~15W (CPU)

- 10-20× speedup on inference

Novelty Angle:

- Adaptive threshold tuning in hardware

- Multi-lead fusion (if you have time)

- Confidence scoring for uncertain cases

Risk Level: ⚠️ LOW

- Model is proven to work

- 1D convolution is simpler than 2D

- Datasets are clean and available

2. Lightweight Crop Disease Detection on Drone Footage ⭐ STRONG CONTENDER

Why This Wins:

- Visual demo (judges can SEE it working)

- India-relevant problem (agricultural impact)

- Edge deployment story is compelling (drones have power constraints)

- Can use transfer learning (MobileNetV2)

Technical Approach:

ML Model:

- MobileNetV2 (pre-trained on ImageNet)

- Fine-tune on PlantVillage dataset

- Input: 96×96 RGB images (small for FPGA)

- Output: Healthy / Disease class

- Quantize to INT8

Hardware Acceleration:

- Accelerate depthwise separable convolutions

- Focus on bottleneck layers

- Tile-based processing (handle larger images)

FPGA Implementation:

Camera Input → Image Preprocessing →

Depthwise Conv Accelerator (YOUR RTL) →

Pointwise Conv → Classification → Disease Alert

Why Judges Love This:

- Scalability story (from lab to real farms)

- Power efficiency matters (battery-powered drones)

- Clear business case (crop loss prevention)

- Societal impact (food security)

Datasets:

- PlantVillage (54K images, 38 disease classes)

- Can subset to 3-5 common diseases

Expected Results:

- Accuracy: ~92-95%

- Latency: 15-25ms per image

- Power: ~3W (FPGA) vs. ~25W (Jetson Nano)

- 8-12× speedup over embedded CPU

Novelty Angle:

- Multi-scale detection (early disease spots are small)

- Uncertainty quantification (don't misdiagnose)

- Progressive inference (fast coarse check, then detailed analysis)

Risk Level: ⚠️ MEDIUM

- Image processing is more complex than 1D signals

- Need to handle variable lighting conditions

- Larger model = more RTL complexity

3. Edge-Based Pothole Detection for Smart Roads

Why This Wins:

- Infrastructure problem (Indian roads need this!)

- Real-time requirement (vehicle moving at speed)

- Can use simple CV + ML hybrid (not pure deep learning)

- Sensor fusion opportunity (accelerometer + camera)

Technical Approach:

ML Model:

- Lightweight object detector (YOLO-Nano or SSD-MobileNet)

- Input: 128×128 grayscale images

- Output: Bounding boxes for potholes

- Quantize to INT8

Hardware Acceleration:

- Accelerate edge detection (Canny/Sobel) in hardware

- Accelerate conv layers for object detection

- Fusion with accelerometer data (vibration patterns)

FPGA Implementation:

Camera → Edge Detection (RTL) →

Conv Accelerator (RTL) →

Detection Head → GPS Logging → Cloud Upload

Why Judges Love This:

- Smart city angle (government priority)

- Multi-sensor fusion (shows system thinking)

- Real-time mapping (valuable dataset creation)

- Safety impact (accident prevention)

Datasets:

- Can create synthetic data (Google Street View + augmentation)

- Or use public dashcam datasets

Expected Results:

- Detection accuracy: ~85-90%

- Latency: <50ms (at 30 FPS)

- False positive rate: <5%

- 15-25× speedup over CPU

Novelty Angle:

- Severity classification (minor crack vs. major pothole)

- Confidence-based alert system

- Crowdsourced road quality mapping

Risk Level: ⚠️ MEDIUM

- Object detection is more complex

- Need good dataset (might require collection)

- Multi-sensor integration adds complexity

🥈 TIER 2: SOLID BACKUP OPTIONS

4. Real-Time Speech Command Recognition for IoT Devices

Why This Works:

- Audio processing (different from typical vision projects)

- Ultra-low power requirement (always-on devices)

- Small model (keyword spotting is lightweight)

Technical Approach:

- Model: Temporal Convolutional Network (TCN) or simple RNN

- Input: MFCC features (39 coefficients × 100 frames)

- Output: 10-20 keywords ("on", "off", "light", etc.)

- Quantize to INT8

Hardware Focus:

- Accelerate MFCC extraction

- Accelerate temporal convolutions

- Streaming architecture (process audio in real-time)

Expected Speedup: 10-15×

Risk Level: ⚠️ LOW-MEDIUM

5. Real-Time Water Quality Monitoring with Sensor Fusion

Why This Works:

- Environmental impact (drinking water safety)

- Multiple sensor inputs (pH, turbidity, temperature, conductivity)

- Anomaly detection (simpler than classification)

Technical Approach:

- Model: Isolation Forest or Autoencoder (anomaly detection)

- Input: Time-series sensor data

- Output: Normal / Contamination alert

- Can use classical ML (Random Forest) + hardware acceleration

Hardware Focus:

- Accelerate decision tree traversal

- Implement autoencoder inference

- Real-time thresholding and alerting

Expected Speedup: 20-30× (classical ML is very FPGA-friendly)

Risk Level: ⚠️ LOW

6. Lightweight Person Re-Identification for Surveillance

Why This Works:

- Security application (surveillance systems)

- Feature extraction focus (not full detection)

- Can use metric learning (Siamese networks)

Technical Approach:

- Model: Lightweight feature extractor (ResNet18 or MobileNet backbone)

- Input: 128×64 person crops

- Output: Feature vector (128-d embedding)

- Compare embeddings for matching

Hardware Focus:

- Accelerate feature extraction

- Hardware-accelerated similarity computation (cosine distance)

Expected Speedup: 12-18×

Risk Level: ⚠️ MEDIUM-HIGH (more complex)

🥉 TIER 3: HIGH RISK, HIGH REWARD

7. Adaptive Traffic Signal Control with RL

Ambitious but impressive if you pull it off:

- Reinforcement learning in hardware

- Real-time traffic optimization

- Tabular Q-learning (FPGA-friendly)

Risk Level: ⚠️⚠️ HIGH (complex, unproven in hardware)

📊 COMPARISON MATRIX

| Project | Win Probability | Technical Difficulty | Demo Impact | Novelty | Time Risk |
| --- | --- | --- | --- | --- | --- |
| ECG Arrhythmia | ⭐⭐⭐⭐⭐ | ⚠️⚠️ | 🎭🎭🎭🎭 | 🔬🔬🔬 | ✅ LOW |
| Crop Disease | ⭐⭐⭐⭐ | ⚠️⚠️⚠️ | 🎭🎭🎭🎭🎭 | 🔬🔬🔬 | ⚠️ MEDIUM |
| Pothole Detection | ⭐⭐⭐⭐ | ⚠️⚠️⚠️ | 🎭🎭🎭🎭 | 🔬🔬🔬🔬 | ⚠️ MEDIUM |
| Speech Commands | ⭐⭐⭐ | ⚠️⚠️ | 🎭🎭🎭 | 🔬🔬 | ✅ LOW |
| Water Quality | ⭐⭐⭐ | ⚠️⚠️ | 🎭🎭🎭 | 🔬🔬🔬 | ✅ LOW |
| Person Re-ID | ⭐⭐⭐ | ⚠️⚠️⚠️⚠️ | 🎭🎭🎭🎭 | 🔬🔬🔬🔬 | ⚠️⚠️ HIGH |

🎯 MY TOP RECOMMENDATION

Go with ECG Arrhythmia Detection

Here's why this is your best bet:

✅ Technical Advantages:

- 1D convolution is MUCH simpler than 2D (images)

  - Fewer multiply-accumulate operations

  - Easier to pipeline

  - Less memory bandwidth required

- Small model size

  - Fits easily in FPGA Block RAM

  - No external DDR needed

  - Faster synthesis and simulation

- Proven architecture

  - Established benchmarks to compare against

  - Clear accuracy targets

  - Well-studied problem

✅ Presentation Advantages:

- Life-critical application

  - Judges immediately understand importance

  - Strong emotional appeal

  - Clear societal impact

- Real-time requirement is GENUINE

  - Not artificial ("we need it fast because... reasons")

  - ICU monitoring actually needs <10ms latency

  - Missed heartbeats can be fatal

- Clean demo

  - Can show live ECG waveform

  - Visual anomaly detection

  - Immediate alert system

✅ Execution Advantages:

- Abundant resources

  - MIT-BIH dataset is standard

  - Tons of research papers to reference

  - Open-source implementations available (for baseline)

- Incremental development

  - Week 1: Train model

  - Week 2: Single conv layer in RTL

  - Week 3: Full pipeline + optimization

- Low risk of failure

  - Even a basic accelerator shows value

  - Can always fall back to simpler version

  - Multiple checkpoints for validation

🛠️ DETAILED TECHNICAL ROADMAP for ECG Project

Week 1: Model Development

Day 1-2: Dataset Setup

- Download MIT-BIH Arrhythmia Database

- Preprocess signals (bandpass filter, normalization)

- Create train/test split (stratified)

Day 3-5: Model Training

# Simple 1D CNN architecture

Input: 180 samples (1 second @ 180 Hz)

↓

Conv1D(32 filters, kernel=5) + ReLU

↓

MaxPool1D(2)

↓

Conv1D(64 filters, kernel=5) + ReLU

↓

MaxPool1D(2)

↓

Dense(64) + ReLU

↓

Dense(2) [Normal / Arrhythmia]

Day 6-7: Quantization

- Quantize to INT8 using PyTorch quantization toolkit

- Validate accuracy (target: <1% degradation)

- Export fixed-point weights

Deliverable: Trained, quantized model with >95% accuracy

Week 2: Hardware Design

Day 8-10: Conv1D Accelerator (Core Module)

// Simplified architecture

module conv1d_accelerator #(

parameter INPUT_WIDTH = 8,

parameter WEIGHT_WIDTH = 8,

parameter KERNEL_SIZE = 5,

parameter NUM_FILTERS = 32

)(

input clk,

input rst,

input [INPUT_WIDTH-1:0] data_in,

input data_valid,

output [INPUT_WIDTH-1:0] data_out,

output data_ready

);

// Systolic array for MAC operations

// Pipelined for throughput

// Double buffering for continuous operation

endmodule

Day 11-12: Integration

- Top-level module connecting layers

- Memory controllers for weight/activation storage

- Control FSM for layer sequencing

Day 13-14: Simulation & Verification

- Create testbench with real ECG data

- Verify against Python golden reference

- Check corner cases

Deliverable: Working RTL passing all testbenches

Week 3: Optimization & Documentation

Day 15-17: Synthesis & Optimization

- Synthesize on target FPGA (Zynq-7020)

- Optimize for timing (target 100 MHz)

- Reduce resource usage if needed

- Generate utilization reports

Day 18-19: Demo Video & Report

- Record simulation waveforms

- Create performance comparison charts

- Write technical report (use template I provided earlier)

- Prepare system architecture diagrams

Day 20-21: Final Polish

- Proofread documentation

- Create backup submission materials

- Prepare FAQ for potential questions

- Practice presentation (if needed)

Deliverable: Complete submission package

📈 EXPECTED PERFORMANCE METRICS

Baseline (Python on CPU):

- Inference time: ~100-150ms per sample

- Power: ~15W (Intel i5)

Your FPGA Implementation:

- Inference time: ~5-10ms per sample

- Power: ~2-3W

- Speedup: 10-15×

- Energy efficiency: 50-75× better

Resource Utilization (Zynq-7020):

- LUTs: ~40-50K (out of 53K) - 75%

- FFs: ~30-40K (out of 106K) - 35%

- DSP48: ~100 (out of 220) - 45%

- BRAM: ~60 (out of 140) - 43%

This shows efficient utilization without overflow.

🎨 DEMO VIDEO OUTLINE

Duration: 7 minutes

0:00-0:30 - Problem Statement

- Show statistics on cardiac deaths

- Explain need for real-time monitoring

0:30-1:30 - Solution Overview

- System architecture diagram

- Explain ML model

- Show FPGA acceleration approach

1:30-3:00 - Technical Deep Dive

- Conv1D accelerator design

- Simulation waveforms

- Synthesis results (resource usage)

3:00-4:30 - Performance Comparison

- Latency comparison (CPU vs FPGA)

- Power consumption

- Accuracy validation

4:30-6:00 - Live Demo

- Real ECG signal input

- Show classification results

- Demonstrate latency (<10ms)

6:00-7:00 - Impact & Future Work

- Deployment scenario (bedside monitor)

- Scalability (multi-lead, more arrhythmias)

- Closing statement

🚨 RISK MITIGATION PLAN

If Conv1D accelerator is too complex: → Fall back to accelerating just the Dense layers (matrix-vector multiply) → Still shows hardware acceleration benefit

If accuracy drops after quantization: → Use INT16 instead of INT8 → Or quantization-aware training

If resource utilization exceeds limit: → Reduce number of filters (32→16) → Time-multiplex operations

If simulation takes too long: → Use smaller testbenches → Verify layer-by-layer instead of end-to-end

✅ FINAL CHECKLIST (Before Submission)

Code:

- [ ] All RTL modules synthesize without errors

- [ ] Testbenches pass with 100% correctness

- [ ] Resource utilization < 80% on target FPGA

- [ ] Timing constraints met (no critical warnings)

- [ ] Code is well-commented and readable

Documentation:

- [ ] Technical report complete (8-12 pages)

- [ ] System architecture diagrams included

- [ ] Performance comparison table with data

- [ ] Synthesis reports attached

- [ ] References cited properly

Demo Video:

- [ ] High quality (1080p minimum)

- [ ] Clear audio (use microphone, not laptop mic)

- [ ] Professional editing (smooth transitions)

- [ ] Under 10 minutes

- [ ] Shows all required elements

Submission Package:

- [ ] All source files organized

- [ ] README with build instructions

- [ ] Video uploaded and link working

- [ ] Report in PDF format

- [ ] Submitted before 15 March deadline

💪 YOU'VE GOT THIS

Why you'll succeed:

- You have strong ML fundamentals (model development will be easy)

- Partner has hardware background (RTL support)

- ECG project is proven and low-risk

- Timeline is realistic with this scope

- Clear differentiation from typical projects

Your winning formula: = Strong ML × Smart hardware partitioning × Clean execution × Professional presentation

Most teams will fail on at least 2 of these. You can nail all 4.

Ready to proceed with ECG Arrhythmia Detection?

I can provide:

- Detailed model architecture (Python code)

- RTL templates for Conv1D accelerator

- Testbench frameworks

- Report structure with section templates

- Demo video script

Let me know if you want to dive deeper into any aspect! 🚀