🔧 Hardware Implementation Guide - For Your Partner

This is what your partner needs to build in Verilog/VHDL.

📋 COMPLETE RTL IMPLEMENTATION CHECKLIST

🎯 What Needs to Be Implemented:

1. ✅ Input Interface & Preprocessing

2. ✅ Conv1D Layer 1 (1→16 channels)

3. ✅ Conv1D Layer 2 (16→32 channels)

4. ✅ Conv1D Layer 3 (32→64 channels)

5. ✅ Global Average Pooling

6. ✅ Fully Connected Layer (64→2)

7. ✅ Output Classification

8. ✅ Memory Management (weight storage)

9. ✅ Control FSM (state machine)

10. ✅ Testbench & Verification

🏗️ DETAILED IMPLEMENTATION ROADMAP

MODULE 1: Top-Level Architecture

System Block Diagram:

┌─────────────────────────────────────────────────────┐

│                ECG ACCELERATOR TOP                  │

│                                                     │

│  ┌─────────┐    ┌──────────┐    ┌──────────┐     │

│  │  Input  │───▶│  Conv1D  │───▶│  Conv1D  │     │

│  │ Buffer  │    │ Layer 1  │    │ Layer 2  │     │

│  └─────────┘    └──────────┘    └──────────┘     │

│                                                     │

│  ┌──────────┐   ┌──────────┐    ┌──────────┐     │

│  │  Conv1D  │───▶│   GAP    │───▶│    FC    │     │

│  │ Layer 3  │    │          │    │  Layer   │     │

│  └──────────┘    └──────────┘    └──────────┘     │

│                                        │            │

│                                        ▼            │

│                                   ┌─────────┐      │

│                                   │ Output  │      │

│                                   │  (0/1)  │      │

│                                   └─────────┘      │

│                                                     │

│  ┌─────────────────────────────────────────┐      │

│  │         Weight Memory (BRAM)            │      │

│  │  Conv1 weights | Conv2 weights | ...   │      │

│  └─────────────────────────────────────────┘      │

│                                                     │

│  ┌─────────────────────────────────────────┐      │

│  │         Control FSM                      │      │

│  └─────────────────────────────────────────┘      │

└─────────────────────────────────────────────────────┘

MODULE 2: Fixed-Point MAC Unit (Building Block)

This is the CORE computation unit - everything else uses this.

What it does:

MAC = Multiply-Accumulate

Result = (Input × Weight) + Accumulator

Verilog Implementation:

module fixed_point_mac #(

parameter WIDTH = 16,        // Q8.8 format

parameter FRAC_BITS = 8

)(

input wire clk,

input wire rst,

input wire enable,

input wire signed [WIDTH-1:0] data_in,

input wire signed [WIDTH-1:0] weight,

input wire accumulate,       // 1=add to accumulator, 0=reset

output reg signed [WIDTH-1:0] mac_out

);

reg signed [2*WIDTH-1:0] mult_result;

reg signed [2*WIDTH-1:0] accumulator;

always @(posedge clk) begin

if (rst) begin

mult_result <= 0;

accumulator <= 0;

mac_out <= 0;

end else if (enable) begin

// Multiply

mult_result <= data_in * weight;

// Accumulate or reset

if (accumulate) begin

accumulator <= accumulator + mult_result;

end else begin

accumulator <= mult_result;

end

// Truncate to output width (keep Q8.8 format)

mac_out <= accumulator[WIDTH+FRAC_BITS-1:FRAC_BITS];

end

end

endmodule

Key Points:

- Uses Q8.8 fixed-point (16 bits total)

- Handles overflow by truncation

- Pipelined for speed

MODULE 3: 1D Convolution Engine

What it does:

- Slides a kernel over input

- Performs MAC operations

- With stride=2, outputs half the input size

Simplified Implementation:

module conv1d_layer #(

parameter INPUT_CHANNELS = 1,

parameter OUTPUT_CHANNELS = 16,

parameter KERNEL_SIZE = 5,

parameter STRIDE = 2,

parameter INPUT_LENGTH = 180

)(

input wire clk,

input wire rst,

input wire start,

input wire signed [15:0] data_in,

input wire data_valid,

output reg signed [15:0] data_out,

output reg data_out_valid,

output reg done

);

// Weight memory (load from BRAM)

reg signed [15:0] weights [0:INPUT_CHANNELS*OUTPUT_CHANNELS*KERNEL_SIZE-1];

// Input buffer (stores kernel_size samples)

reg signed [15:0] input_buffer [0:KERNEL_SIZE-1];

// MAC units (instantiate multiple for parallel processing)

wire signed [15:0] mac_result;

fixed_point_mac mac_unit (

.clk(clk),

.rst(rst),

.enable(mac_enable),

.data_in(input_buffer[mac_index]),

.weight(weights[weight_index]),

.accumulate(accumulate),

.mac_out(mac_result)

);

// Control logic (FSM)

// States: IDLE, LOAD_INPUT, COMPUTE, OUTPUT

// ... (implement state machine here)

endmodule

Optimization Ideas:

- Parallel MAC units - Use 4-8 MACs in parallel

- Pipelining - Multi-stage pipeline for throughput

- Weight reuse - Cache weights in registers

MODULE 4: Global Average Pooling

What it does:

- Takes (64, 23) input

- Averages each of 64 channels

- Outputs (64, 1)

Implementation:

module global_avg_pool #(

parameter CHANNELS = 64,

parameter INPUT_LENGTH = 23

)(

input wire clk,

input wire rst,

input wire [15:0] data_in,

input wire data_valid,

output reg [15:0] data_out,

output reg data_out_valid

);

reg [31:0] accumulators [0:CHANNELS-1];

reg [4:0] sample_count;

reg [5:0] channel_count;

always @(posedge clk) begin

if (rst) begin

sample_count <= 0;

channel_count <= 0;

end else if (data_valid) begin

// Accumulate

accumulators[channel_count] <= accumulators[channel_count] + data_in;

sample_count <= sample_count + 1;

if (sample_count == INPUT_LENGTH - 1) begin

// Divide by INPUT_LENGTH (can use bit shift if power of 2)

data_out <= accumulators[channel_count] / INPUT_LENGTH;

data_out_valid <= 1;

channel_count <= channel_count + 1;

sample_count <= 0;

end

end

end

endmodule

Optimization:

- If INPUT_LENGTH is power of 2, use bit shift instead of division

- For 23, approximate with shift: >> 5 (divide by 32, close enough)

MODULE 5: Fully Connected Layer

What it does:

- Matrix-vector multiplication

- 64 inputs → 2 outputs

- Classification layer

Implementation:

module fully_connected #(

parameter INPUT_SIZE = 64,

parameter OUTPUT_SIZE = 2

)(

input wire clk,

input wire rst,

input wire start,

input wire signed [15:0] data_in [0:INPUT_SIZE-1],

output reg signed [15:0] data_out [0:OUTPUT_SIZE-1],

output reg done

);

// Weights: [OUTPUT_SIZE][INPUT_SIZE]

reg signed [15:0] weights [0:OUTPUT_SIZE*INPUT_SIZE-1];

// MAC for each output

integer i, j;

reg signed [31:0] accumulator [0:OUTPUT_SIZE-1];

always @(posedge clk) begin

if (rst) begin

done <= 0;

end else if (start) begin

// Compute each output

for (i = 0; i < OUTPUT_SIZE; i = i + 1) begin

accumulator[i] = 0;

for (j = 0; j < INPUT_SIZE; j = j + 1) begin

accumulator[i] = accumulator[i] +

(data_in[j] * weights[i*INPUT_SIZE + j]);

end

data_out[i] <= accumulator[i][23:8]; // Truncate to Q8.8

end

done <= 1;

end

end

endmodule

Note: This is simplified. Real implementation should pipeline this!

MODULE 6: Weight Memory Management

What it does:

- Stores all model weights

- Provides weights to compute units

Memory Requirements:

Conv1: 1 × 16 × 5 = 80 weights

Conv2: 16 × 32 × 5 = 2,560 weights

Conv3: 32 × 64 × 5 = 10,240 weights

FC: 64 × 2 = 128 weights

Total: 13,008 weights × 16 bits = 26 KB

Implementation:

module weight_memory #(

parameter ADDR_WIDTH = 14,   // 2^14 = 16K addresses

parameter DATA_WIDTH = 16

)(

input wire clk,

input wire [ADDR_WIDTH-1:0] addr,

output reg [DATA_WIDTH-1:0] weight

);

// Block RAM

(* ram_style = "block" *) reg [DATA_WIDTH-1:0] mem [0:(1<<ADDR_WIDTH)-1];

// Load weights from file during synthesis

initial begin

$readmemh("weights.hex", mem);

end

always @(posedge clk) begin

weight <= mem[addr];

end

endmodule

MODULE 7: Control FSM

What it does:

- Orchestrates the entire pipeline

- Sequences layer execution

- Manages data flow

State Machine:

module control_fsm (

input wire clk,

input wire rst,

input wire start,

output reg [2:0] state,

output reg conv1_enable,

output reg conv2_enable,

output reg conv3_enable,

output reg gap_enable,

output reg fc_enable,

output reg done

);

// States

localparam IDLE = 3'd0;

localparam CONV1 = 3'd1;

localparam CONV2 = 3'd2;

localparam CONV3 = 3'd3;

localparam GAP = 3'd4;

localparam FC = 3'd5;

localparam DONE = 3'd6;

always @(posedge clk) begin

if (rst) begin

state <= IDLE;

done <= 0;

end else begin

case (state)

IDLE: begin

if (start) state <= CONV1;

end

CONV1: begin

conv1_enable <= 1;

if (conv1_done) begin

conv1_enable <= 0;

state <= CONV2;

end

end

CONV2: begin

conv2_enable <= 1;

if (conv2_done) begin

conv2_enable <= 0;

state <= CONV3;

end

end

CONV3: begin

conv3_enable <= 1;

if (conv3_done) begin

conv3_enable <= 0;

state <= GAP;

end

end

GAP: begin

gap_enable <= 1;

if (gap_done) begin

gap_enable <= 0;

state <= FC;

end

end

FC: begin

fc_enable <= 1;

if (fc_done) begin

fc_enable <= 0;

state <= DONE;

end

end

DONE: begin

done <= 1;

state <= IDLE;

end

endcase

end

end

endmodule

MODULE 8: Testbench

What it does:

- Feeds test vectors

- Captures outputs

- Compares against golden reference

Basic Testbench:

module tb_ecg_accelerator;

reg clk, rst, start;

reg [15:0] data_in;

wire result;

wire done;

// DUT

ecg_accelerator_top dut (

.clk(clk),

.rst(rst),

.start(start),

.data_in(data_in),

.result(result),

.done(done)

);

// Clock generation

always #5 clk = ~clk;

// Test vectors

reg [15:0] test_inputs [0:8999];  // 50 cases × 180 samples

reg expected_outputs [0:49];

initial begin

$readmemh("inputs.hex", test_inputs);

$readmemh("outputs.hex", expected_outputs);

clk = 0;

rst = 1;

#20 rst = 0;

// Run tests

for (int i = 0; i < 50; i++) begin

start = 1;

#10 start = 0;

// Feed 180 samples

for (int j = 0; j < 180; j++) begin

data_in = test_inputs[i*180 + j];

#10;

end

// Wait for done

wait(done);

// Check result

if (result == expected_outputs[i]) begin

$display("Test %d: PASS", i);

end else begin

$display("Test %d: FAIL (expected %d, got %d)",

i, expected_outputs[i], result);

end

end

$finish;

end

endmodule

📊 IMPLEMENTATION STRATEGY (For Your Partner)

Phase 1: Build & Test Basic Blocks (Days 6-7)

- ✅ Fixed-point MAC unit

- ✅ Test MAC with known values

- ✅ Weight memory module

- ✅ Test weight loading

Phase 2: Build Layers (Days 8-9)

- ✅ Conv1D engine (single channel first)

- ✅ Test with simple patterns

- ✅ Expand to multi-channel

- ✅ Add GAP module

- ✅ Add FC module

Phase 3: Integration (Day 10)

- ✅ Connect all modules

- ✅ Implement control FSM

- ✅ Create top-level wrapper

Phase 4: Verification (Days 11-12)

- ✅ Run testbench with test vectors

- ✅ Debug mismatches

- ✅ Achieve 98% accuracy

Phase 5: Optimization (Day 13)

- ✅ Pipeline for speed

- ✅ Optimize resource usage

- ✅ Meet timing constraints

🎯 TARGET SPECIFICATIONS

| Metric | Target | How to Achieve |
| --- | --- | --- |
| Latency | <10ms | Pipeline, parallel MACs |
| Throughput | >100 inf/sec | Streaming architecture |
| Clock Frequency | 100 MHz | Proper pipelining |
| Resource Usage | <80% | Efficient memory, shared MACs |
| Power | <3W | Clock gating, minimal logic |
| Accuracy | 98% | Match fixed-point exactly |

🔧 TOOLS YOUR PARTNER NEEDS:

- Vivado (Xilinx) or Quartus (Intel/Altera)

- ModelSim or Vivado Simulator

- Python (for verification scripts)

📚 HELPFUL RESOURCES FOR YOUR PARTNER:

Learning Resources:

- Xilinx FPGA Tutorial: https://www.xilinx.com/support/university.html

- Verilog Tutorial: https://www.asic-world.com/verilog/

- Fixed-Point Arithmetic: https://en.wikipedia.org/wiki/Fixed-point_arithmetic

Similar Projects (Reference):

- FINN Framework (Xilinx): https://github.com/Xilinx/finn

- Haddoc2 (CNN on FPGA): https://github.com/DreamIP/haddoc2

✅ SUMMARY FOR YOUR PARTNER:

What to build:

- MAC unit (core building block)

- 3× Conv1D layers

- Global Average Pooling

- 1× FC layer

- Control FSM

- Weight memory

- Testbench

How to verify:

- Use inputs.hex and outputs.hex

- Target: 49/50 correct (98%)

Timeline:

- Days 6-9: Build modules

- Days 10-11: Integration & testing

- Day 12-13: Optimization

Want me to create a detailed handoff document with this info? 📄